SMART PLANT WATERING SYSTEM

With Real-Time Mobile Monitoring Application

Submitted by:

CATLI, Bernald F.

CASTRO, Maryann C.

DOMINGO, Edward John G.

MERCADO, Martin Eric P.

ROBLES, Raven C.

BSCS3A

Submitted to:

Mr. Patrick James Torres

Elective 2

FINALS 2026

ABSTRACT

This document presents the design, implementation, and operation of a Smart Plant Watering System utilizing an ESP32 microcontroller integrated with an inductive proximity metal sensor and a Flutter-based mobile application. The system monitors water levels via a metal-float mechanism, providing immediate local feedback through an OLED display, buzzer, and LED, while simultaneously transmitting status updates to a mobile interface. This project demonstrates an automated agricultural solution designed to improve plant maintenance efficiency through IoT (Internet of Things) integration.

SYSTEM OVERVIEW

The Smart Plant Watering System is an IoT-based monitoring unit powered by an ESP32. It serves as a continuous supervisor for water reservoirs, using high-frequency sensing to prevent dry-run conditions.

FUNCTIONAL FLOW

Water Level Drop → Metal Float Reaches Sensor → Inductive Signal Triggered → ESP32 Processing → Local Alert (Buzzer/LED/OLED) → Mobile Notification via Firebase.

SYSTEM FEATURES

- Non-contact water level detection using inductive proximity.

- Real-time local status display via I2C OLED.

- Audible and visual emergency alerts (Buzzer and LED).

- Remote monitoring via a dedicated Flutter application.

- Low-latency data synchronization using Firebase Realtime Database.

HARDWARE DESIGN AND COMPONENTS

COMPONENTS LIST

- ESP32 Development Board: Core microcontroller with built-in Wi-Fi.

- Inductive Proximity Metal Sensor: Detects the presence of the metal float.

- 0.96" OLED Display (I2C): Displays system status and water levels.

- Passive Buzzer: Provides audible alerts when water is low.

- LED (Red/Green): Provides immediate visual status.

- Resistors (10kΩ and 20kΩ): Used for the essential voltage divider circuit.

- Breadboard and Jumper Wires: For prototype assembly.

POWER CONSIDERATIONS

- The Inductive Sensor typically requires 5V to 12V (Powered via ESP32 Vin).

- The ESP32 logic pins operate at 3.3V; a voltage divider is required for the sensor signal.

- Common ground must be maintained across all components for signal stability.

FIRMWARE IMPLEMENTATION

DEVELOPMENT ENVIRONMENT

- Arduino IDE (Version 2.x)

- ESP32 Core Framework

- Required Libraries

- Wire.h (I2C communication)

- Adafruit_GFX & Adafruit_SSD1306 (OLED Control)

- Firebase_ESP_Client (Cloud connectivity)

CORE FUNCTIONALITIES

- Digital Signal Processing: Reading the state of the proximity sensor on GPIO 34.

- Alert Logic: Triggering the PWM signal for the buzzer and digital high for the LED.

- OLED Rendering: Dynamic UI updates showing "LEVEL OK" or "REFILL REQUIRED.

MOBILE APPLICATION DESIGN

The mobile application is developed using the Flutter framework and serves as the primary monitoring interface for the Smart Plant Watering System. Unlike traditional offline systems, this application utilizes Firebase Realtime Database to enable remote monitoring. This allows users to receive water level updates even when they are not connected to the same local Wi-Fi network as the ESP32.

FEATURES

- Real-time water level status dashboard.

- Visual warning indicators (Color-coded UI).

- Historical log of "Low Water" events.

- Push notifications for critical alerts.

SYSTEM SETUP AND OPERATION

INITIAL SETUP

- Assemble the circuit on the breadboard following the voltage divider schematic.

- Mount the inductive sensor at the "Low" threshold of the water tank.

- Attach a metal disc to a floatation device (cork/foam) inside the tank.

- Flash the ESP32 with the provided firmware.

SYSTEM OPERATION

- Normal State: The metal float is away from the sensor. OLED shows "GOOD," LED is off.

- Low Water State: The float drops; the sensor detects the metal disc.

- Alert State: ESP32 triggers the buzzer and LED; the Flutter app displays a red warning screen.

DATA DOCUMENTATION

Firebase Path Structure:

- /sensor_status: Integer (0 = OK, 1 = Low Water)

- /last_refill: Timestamp

- /system_uptime: String

CONCLUSION

This Smart Plant Watering System prototype provides a scalable solution for automated plant care. By integrating the ESP32 with a robust inductive sensing method and a modern Flutter UI, the project meets the academic requirements for Elective 2 at Concepcion Holy Cross College, Inc. while demonstrating practical IoT application.